1. Register Transfer Operations and ALU Operations

Aim

To design and simulate a system that performs register transfer operations and basic arithmetic and logical operations using multiple registers and an ALU.

Components Required

- Registers: R0, R1, R2, R3

- ALU

- Control unit

- Multiplexer

- Common data bus

- Clock

- Memory unit

- Input/Output unit

Register Organization

Assume each register is 8 bits wide.

┌──────────┐

R0 ────────►│          │

R1 ────────►│   ALU    │──────► Result

R2 ────────►│          │

R3 ────────►│          │

└──────────┘

▲

│

Control Unit

The registers communicate with the ALU through a common data path.

Register Transfer Operations

Register transfer notation:

- R1 ← R0 : Transfer contents of R0 to R1.

- R2 ← R1 : Transfer contents of R1 to R2.

- R3 ← R1 + R2 : Add R1 and R2 and store the result in R3.

Example

Initially:

| Register | Value |
| --- | --- |
| R0 | 10 |
| R1 | 5 |
| R2 | 0 |
| R3 | 0 |

Operation 1: Transfer

R2 ← R0

After execution:

| Register | Value |
| --- | --- |
| R0 | 10 |
| R1 | 5 |
| R2 | 10 |
| R3 | 0 |

Operation 2: Addition

R3 ← R0 + R1

The ALU receives 10 and 5.

10 + 5 = 15

Therefore:

R3 = 15

Operation 3: Subtraction

R3 ← R0 - R1

10 - 5 = 5

Therefore:

R3 = 5

Operation 4: AND

R3 ← R0 AND R1

For example:

R0 = 1010

R1 = 0101

AND = 0000

Therefore:

R3 = 0

Operation 5: OR

R0 = 1010

R1 = 0101

OR = 1111

Therefore:

R3 = 15

ALU Role

The ALU performs the actual arithmetic and logical processing.

| Operation | ALU Function |
| --- | --- |
| Addition | A + B |
| Subtraction | A − B |
| AND | A AND B |
| OR | A OR B |

The control unit selects the required ALU operation and determines which registers provide the operands and where the result is stored.

Sequence of Operations

R0 → R2

R0 + R1 → R3

R0 - R1 → R3

R0 AND R1 → R3

R0 OR R1 → R3

Result

The system successfully demonstrates register-to-register transfers and arithmetic/logic operations. The ALU acts as the central processing unit for arithmetic and logical operations, while the registers temporarily store operands and results.

2. Single-Bus and Multi-Bus Organization

Aim

To model and compare single-bus and multi-bus computer organizations for transferring data between registers, memory, and I/O devices.

Single-Bus Organization

In a single-bus system, all registers and functional units share one common bus.

R0 ─┐

R1 ─┤

R2 ─┤

R3 ─┤

▼

┌─────────┐

│ Common  │

│   Bus   │

└────┬────┘

│

┌────┴────┐

│   ALU   │

└────┬────┘

│

Memory

│

I/O

Only one major transfer can use the bus at a time.

Example

To perform:

R3 ← R1 + R2

The sequence may be:

T1: R1 → Bus

T2: Bus → ALU input A

T3: R2 → Bus

T4: Bus → ALU input B

T5: ALU performs addition

T6: ALU result → Bus

T7: Bus → R3

This requires several clock cycles.

Multi-Bus Organization

A multi-bus system uses two or more buses simultaneously.

A typical three-bus architecture contains:

┌─────────┐

R1 ─────►│         │

│  Bus A  │─────┐

R2 ─────►│         │     │

└─────────┘     ▼

┌───────┐

│  ALU  │

└───┬───┘

│

┌─────────┐      │

│  Bus B  │◄─────┘

└────┬────┘

│

▼

R3

Two operands can be supplied to the ALU simultaneously.

For:

R3 ← R1 + R2

Bus A = R1

Bus B = R2

ALU = ADD

R3 ← ALU result

The operation can potentially be completed in a single datapath cycle.

Comparison

| Feature | Single Bus | Multi Bus |
| --- | --- | --- |
| Number of buses | 1 | 2 or more |
| Hardware complexity | Low | Higher |
| Cost | Lower | Higher |
| Parallel transfers | Limited | High |
| Bus contention | High | Reduced |
| Performance | Lower | Higher |
| Control complexity | Simple | More complex |
| Data throughput | Lower | Higher |

Example Timing

Assume a single-bus implementation requires 7 clock cycles for an operation and a multi-bus implementation requires 3 cycles.

Then:

Single Bus = 7 cycles

Multi Bus  = 3 cycles

Approximate speedup:

Result

The multi-bus architecture provides better performance because multiple data transfers can occur concurrently. It reduces bus contention and datapath bottlenecks, although it requires additional hardware and more complex control logic.

3. Five-Stage Instruction Pipeline

Aim

To simulate a five-stage instruction pipeline and compare its performance with a non-pipelined processor.

Pipeline Stages

The five stages are:

- IF – Instruction Fetch

- ID – Instruction Decode

- EX – Execute

- MEM – Memory Access

- WB – Write Back

Instruction

│

▼

┌──────┐

│  IF  │

└──┬───┘

▼

┌──────┐

│  ID  │

└──┬───┘

▼

┌──────┐

│  EX  │

└──┬───┘

▼

┌──────┐

│ MEM  │

└──┬───┘

▼

┌──────┐

│  WB  │

└──────┘

Example Instructions

Consider four instructions:

I1: ADD R1, R2, R3

I2: SUB R4, R5, R6

I3: AND R7, R8, R9

I4: OR  R10, R11, R12

Pipeline Timing

| Cycle | I1 | I2 | I3 | I4 |
| --- | --- | --- | --- | --- |
| 1 | IF |  |  |  |
| 2 | ID | IF |  |  |
| 3 | EX | ID | IF |  |
| 4 | MEM | EX | ID | IF |
| 5 | WB | MEM | EX | ID |
| 6 |  | WB | MEM | EX |
| 7 |  |  | WB | MEM |
| 8 |  |  |  | WB |

Thus, the pipeline completes four instructions in 8 cycles.

Non-Pipelined Execution

If each instruction requires all five stages sequentially:

I1 = 5 cycles

I2 = 5 cycles

I3 = 5 cycles

I4 = 5 cycles

Total:

Speedup

Pipeline:

where:

- k = 5 stages

- n = 4 instructions

Therefore:

Non-pipelined:

Speedup:

Throughput

For a sufficiently long instruction sequence, a five-stage pipeline can ideally complete approximately one instruction per cycle after the pipeline is filled.

Therefore:

Non-pipelined throughput ≈ 1 instruction / 5 cycles

Pipelined throughput ≈ 1 instruction / cycle

Result

Pipelining improves instruction throughput by overlapping different stages of multiple instructions. The main advantage is increased instruction-level parallelism, although hazards and pipeline stalls can reduce the ideal performance.

4. Pipeline Hazards and Hazard Resolution

Aim

To simulate data, control, and structural hazards and apply stalling, forwarding, and branch prediction techniques.

Types of Hazards

1. Data Hazard

Occurs when one instruction depends on the result of an earlier instruction.

Example:

I1: ADD R1, R2, R3

I2: SUB R4, R1, R5

I2 needs R1, but I1 has not completed writing it yet.

Without Forwarding

The processor must insert stalls.

Cycle:  1   2   3   4   5   6

I1:     IF  ID  EX  MEM WB

I2:         IF  ID  ST  EX  MEM

The stall increases execution time.

With Data Forwarding

The result of the EX stage can be directly forwarded to the next instruction's EX stage.

I1: ADD ──────────────┐

│ Forward

▼

I2: SUB ─────────────► EX

This reduces or eliminates unnecessary stalls.

2. Control Hazard

A control hazard occurs because the processor does not immediately know which instruction to execute after a branch.

Example:

I1: BEQ R1, R2, TARGET

I2: ADD R3, R4, R5

I3: SUB R6, R7, R8

If the branch is taken, I2 and possibly I3 may have been fetched incorrectly.

Solution: Branch Prediction

The processor predicts whether the branch will be:

Taken

or

Not Taken

If the prediction is correct, pipeline execution continues without a major interruption.

If the prediction is incorrect, incorrectly fetched instructions are flushed.

3. Structural Hazard

Occurs when two pipeline stages require the same hardware resource simultaneously.

Example:

Instruction 1 → Memory access

Instruction 2 → Instruction fetch

If the processor has only one memory unit, both operations cannot occur simultaneously.

Solution

Possible solutions include:

- Separate instruction and data caches

- Additional hardware resources

- Pipeline stalling

- Resource scheduling

Hazard Comparison

| Hazard | Cause | Solution |
| --- | --- | --- |
| Data | Instruction dependency | Forwarding/stalling |
| Control | Branch/jump | Branch prediction |
| Structural | Resource conflict | Extra hardware/stalling |

Performance Example

Suppose 100 instructions require:

Ideal cycles = 100 + 4 = 104 cycles

Suppose hazards introduce:

10 data-hazard stalls

5 branch penalties

3 structural stalls

Total:

With forwarding and branch prediction, assume only:

3 data stalls

2 branch penalties

1 structural stall

Then:

Thus, hazard resolution reduces execution time from 122 cycles to 110 cycles.

Result

Pipeline hazards reduce the theoretical performance of a processor. Forwarding reduces data stalls, branch prediction reduces control-hazard penalties, and additional hardware resources can reduce structural hazards.

5. Superscalar, Out-of-Order, Speculative Execution and SMT

Aim

To analyze a processor capable of executing multiple instructions simultaneously using superscalar, out-of-order, speculative execution, and simultaneous multithreading.

Superscalar Execution

A superscalar processor has multiple execution units and can issue multiple instructions per clock cycle.

Example:

┌──► ALU 1

Instruction ────┼──► ALU 2

Queue           ├──► Load/Store

└──► FPU

For a 4-wide superscalar processor, up to four instructions can potentially be issued per cycle.

Example:

Cycle 1: I1 I2 I3 I4

Cycle 2: I5 I6 I7 I8

Cycle 3: I9 I10 I11 I12

The actual throughput depends on dependencies, available functional units, memory latency, and branch behavior.

Out-of-Order Execution

In-order execution:

I1 → I2 → I3 → I4

If I2 is waiting for memory, later independent instructions may also be delayed.

With out-of-order execution:

I1 → Waiting

I2 → Execute

I3 → Execute

I4 → Execute

Independent instructions can execute before an earlier stalled instruction.

Example

I1: LOAD R1, [Memory]

I2: ADD R4, R5, R6

I3: AND R7, R8, R9

If I1 experiences a memory delay, I2 and I3 can execute while I1 is waiting.

Speculative Execution

The processor predicts future execution paths, particularly branch outcomes.

Example:

Branch

│

┌───┴───┐

▼       ▼

Path A   Path B

│

Prediction

The processor executes instructions on the predicted path before the branch is completely resolved.

If the prediction is correct:

No major delay

If incorrect:

Speculative instructions are discarded

Hyper-Threading and SMT

Simultaneous Multithreading (SMT) allows multiple instruction streams or threads to share the resources of a single physical processor core.

For example:

Physical Core

│

┌─────────┴─────────┐

│                   │

Thread 1            Thread 2

│                   │

└─────────┬─────────┘

▼

Shared Execution

Resources

Intel's implementation of SMT is commonly known as Hyper-Threading.

Why SMT Improves Utilization

Suppose Thread 1 is waiting for memory:

Thread 1 → Memory stall

Instead of leaving execution resources idle:

Thread 2 → Uses available execution units

Therefore, processor resources are utilized more effectively.

Performance Comparison

Assume a processor executes 100 instructions.

Single-Issue Processor

Approximately:

1 instruction/cycle

Ideal execution:

100 cycles

2-Wide Superscalar

Ideally:

2 instructions/cycle

Approximate execution:

50 cycles

4-Wide Superscalar

Ideally:

4 instructions/cycle

Approximate execution:

25 cycles

These are idealized values; real processors achieve lower IPC because of dependencies, cache misses, branch mispredictions, limited execution resources, and other bottlenecks.

Comparison

| Feature | Basic Pipeline | Superscalar | Out-of-Order + Speculative | SMT |
| --- | --- | --- | --- | --- |
| Multiple instructions | Limited | Yes | Yes | Yes |
| Multiple execution units | Not necessary | Yes | Yes | Shared |
| Out-of-order execution | No | Optional | Yes | Usually supported |
| Branch prediction | Basic/optional | Common | Important | Important |
| Resource utilization | Moderate | High | Very high | Higher |
| Instruction throughput | Moderate | High | Very high | Higher under suitable workloads |
| Hardware complexity | Low | High | Very high | Very high |

Overall Result

Superscalar execution increases instruction-level parallelism by issuing multiple instructions per cycle. Out-of-order execution allows independent instructions to bypass stalled instructions, while speculative execution hides branch latency through prediction. SMT improves utilization by allowing multiple threads to share otherwise idle execution resources.

However, performance does not scale linearly with issue width because dependencies, branch mispredictions, memory latency, cache misses, and limited functional-unit availability constrain the achievable IPC (Instructions Per Cycle).

Overall Conclusion

The five simulations demonstrate the progressive development of processor performance:

Register Transfer

↓

Single/Multi-Bus Datapath

↓

Instruction Pipelining

↓

Hazard Handling

↓

Superscalar + Out-of-Order

↓

Speculative Execution + SMT

Each technique addresses a different processor bottleneck:

| Technique | Main Improvement |
| --- | --- |
| Register Transfer + ALU | Data processing |
| Multi-Bus | Faster data movement |
| Pipelining | Higher instruction throughput |
| Hazard Resolution | Reduces pipeline stalls |
| Superscalar | Multiple instructions per cycle |
| Out-of-Order | Better execution-unit utilization |
| Speculative Execution | Reduces branch delays |
| SMT/Hyper-Threading | Better resource utilization across threads |

Final observation: Modern processors combine these techniques rather than using them independently. The resulting architecture achieves high instruction throughput by exploiting data-level, instruction-level, and thread-level parallelism while minimizing the effects of dependencies, branches, and resource conflicts.