NAME : ROSHAN K

REG NO: 192425014
DEPARTMENT: AI & ML

DATE : 03-08-2026

1. Signed 8-bit Arithmetic Using 2's Complement (+45 and −23)

Step 1: Convert Decimal to Binary

+45

45 = 00101101

−23

First convert +23:

23 = 00010111

Find 2's complement:

1's complement = 11101000

Add 1          = 11101001

So,

−23 = 11101001

(A) Addition: +45 + (−23)

00101101   (+45)

+ 11101001   (−23)

------------

00010110

Discard the carry out.

Result = 00010110 = +22

Verification

45 + (−23) = 22 ✓

Overflow Check

Overflow occurs only if:

- Positive + Positive = Negative

- Negative + Negative = Positive

Here,

Positive + Negative

Overflow does not occur.

Another method:

Carry into MSB = Carry out of MSB

Since both are equal,

Overflow = No

(B) Subtraction: +45 − (−23)

Subtracting a negative means adding its positive.

Find +23

00010111

Now,

00101101

+ 00010111

------------

01000100

01000100 = 68

Verification

45 − (−23) = 68 ✓

Overflow Check

Operands are positive.

Result is also positive.

No overflow

Processor Overflow Detection

A processor checks overflow using

Overflow = Carry into MSB XOR Carry out of MSB

If XOR = 1

Overflow occurred.

Otherwise,

No overflow.

Final Answers

Addition

+45 + (−23) = +22

Binary = 00010110

Overflow = No

Subtraction

+45 − (−23) = +68

Binary = 01000100

Overflow = No

2. 4-bit Carry Look-Ahead Adder (CLA)

Assume

A = 1011

B = 0110

Cin = 0

Step 1: Generate (G) and Propagate (P)

Formula

Gi = Ai × Bi

Pi = Ai ⊕ Bi

| Bit | A | B | G | P |
| --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 |
| 2 | 0 | 1 | 0 | 1 |
| 3 | 1 | 0 | 0 | 1 |

Step 2: Carry Equations

C1 = G0 + P0C0

C2 = G1 + P1G0 + P1P0C0

C3 = G2 + P2G1 + P2P1G0 + P2P1P0C0

C4 = G3 + P3G2 + P3P2G1 + P3P2P1G0 + P3P2P1P0C0

All carries are calculated simultaneously.

Step 3: Sum

Si = Pi XOR Ci

Ripple Carry Delay

Carries move one stage at a time.

Bit0 → Bit1 → Bit2 → Bit3

Delay

≈ 4 gate delays

CLA Delay

Carries computed together.

Delay ≈ 2 gate delays

Comparison

| Ripple Carry | Carry Look Ahead |
| --- | --- |
| Carry generated sequentially | Carry generated simultaneously |
| Slow | Fast |
| Simple hardware | Complex hardware |
| Large delay | Very small delay |

Why CLA is Faster?

- Carries are predicted before addition.

- No waiting for previous carry.

- Used in modern CPUs for high-speed arithmetic.

3. Booth's Algorithm (−6 × +5)

Multiplicand

M = −6 = 1010

Multiplier

Q = +5 = 0101

Initialize

A = 0000

Q = 0101

Q−1 = 0

Rules

| Q0Q−1 | Operation |
| --- | --- |
| 00 | No operation |
| 11 | No operation |
| 01 | A = A + M |
| 10 | A = A − M |

After every step

Arithmetic Right Shift

Cycle 1

Q0Q−1 = 10

A = A − M

Shift

Cycle 2

Q0Q−1 = 11

No operation

Shift

Cycle 3

Q0Q−1 = 01

A = A + M

Shift

Cycle 4

Q0Q−1 = 10

A = A − M

Shift

Final Result

11100010

Decimal

−30

Verification

−6 × 5 = −30 ✓

Advantages

- Handles positive and negative numbers directly.

- Reduces additions for consecutive 1s.

- Faster than ordinary shift-add multiplication.

4. Division (13 ÷ 3)

Dividend = 1101

Divisor = 0011

Expected

Quotient = 4

Remainder = 1

(A) Restoring Division

Algorithm

- Shift left

- Subtract divisor

- If remainder negative

  - Restore remainder

  - Quotient bit = 0

- Else

  - Quotient bit = 1

Result

Quotient = 0100 (4)

Remainder = 0001 (1)

(B) Non-Restoring Division

Algorithm

- Shift left

- If previous remainder positive

  - Subtract divisor

- If previous remainder negative

  - Add divisor

- Restoration only once at the end (if needed)

Result

Quotient = 0100

Remainder = 0001

Comparison

| Restoring | Non-Restoring |
| --- | --- |
| Restores remainder every negative step | No immediate restoration |
| More arithmetic operations | Fewer operations |
| Slower | Faster |
| Simple logic | More efficient |

Why Non-Restoring is Preferred?

- Eliminates repeated restore operations.

- Fewer additions/subtractions.

- Higher execution speed.

- Widely used in modern processors.

5. IEEE 754 Floating Point (−13.25)

Step 1: Decimal to Binary

13 = 1101

0.25 = 0.01

Therefore

13.25 = 1101.01₂

Normalize

1.10101 × 2³

Single Precision (32-bit)

Sign

Negative

S = 1

Exponent

Bias = 127

3 + 127 = 130

130 = 10000010

Mantissa

Remove leading 1

10101000000000000000000

Final

1

10000010

10101000000000000000000

Double Precision (64-bit)

Bias

1023

Exponent

3 + 1023 = 1026

1026 = 10000000010

Mantissa

1010100000000000000000000000000000000000000000000000

Final

1

10000000010

1010100000000000000000000000000000000000000000000000

Floating Point Addition

Example

13.25 + 6.5

Step 1

Convert both to IEEE format.

Step 2

Compare exponents.

Step 3

Shift mantissa of smaller exponent.

Step 4

Add mantissas.

Step 5

Normalize result.

Step 6

Round according to IEEE 754 (typically "round to nearest, ties to even").

Step 7

Store sign, exponent, and mantissa.

Precision Issues

Normalization

Ensures the number is stored in the standard form:

1.xxxxx × 2^n

Rounding

Extra bits beyond the mantissa are rounded, which may slightly change the stored value.

Precision Loss

Many decimal fractions (e.g., 0.1) cannot be represented exactly in binary, leading to small approximation errors. Repeated floating-point operations can accumulate these errors.