NAME : ROSHAN K

REG NO: 192425014

DEPARTMENT: AI &amp; ML

DATE : 03-08-2026

1. Debugging Signed Integer Addition (2's Complement Overflow)

Problem

A program adds +120 and +50 using 8-bit 2's complement.

Step 1: Convert to Binary

+120 = 01111000

+50  = 00110010

Step 2: Perform Addition

01111000

+  00110010

------------

10101010

Binary result:

10101010

Interpreted as a signed 8-bit number:

10101010 = -86

Expected mathematical result:

120 + 50 = 170

Since the 8-bit signed range is:

-128 to +127

170 cannot be represented.

Overflow Detection

Overflow occurs because:

- Both operands are positive

- Result becomes negative

Another detection method:

Overflow = Carry into MSB ⊕ Carry out of MSB

If XOR = 1, overflow has occurred.

Debugging Result

Cause:

- Arithmetic overflow due to limited 8-bit range.

Solution

- Check overflow after every signed addition.

- If overflow occurs:

  - Display an overflow error.

  - Use a larger data type (16-bit or 32-bit integer).

  - Prevent incorrect signed interpretation.

2. Debugging Ripple Carry Adder Delay

Problem

A ripple carry adder is slow because each carry must propagate through every bit.

Carry Propagation

Example:

A = 1011

B = 0110

Carry flow:

Bit0 → Bit1 → Bit2 → Bit3

Each bit waits for the previous carry.

If one full adder delay is T, then

Total delay = 4T

For larger adders:

32-bit → 32T

64-bit → 64T

Delay becomes significant.

Performance Bottleneck

The critical path is the carry chain.

Every stage must wait before computing the next carry.

Optimized Solution: Carry Look-Ahead Adder (CLA)

CLA computes carries in parallel.

Generate signal:

Gi = Ai · Bi

Propagate signal:

Pi = Ai ⊕ Bi

Carry equations:

C1 = G0 + P0C0

C2 = G1 + P1G0 + P1P0C0

C3 = G2 + P2G1 + P2P1G0 + P2P1P0C0

Carries are available simultaneously.

Improvement

| Ripple Carry Adder | Carry Look-Ahead Adder |
| --- | --- |
| Sequential carry propagation | Parallel carry computation |
| High delay | Low delay |
| Simple hardware | More hardware |
| Slow for large adders | Fast for processors |

3. Debugging Booth's Multiplication

Problem

Booth's algorithm gives incorrect results for negative numbers.

Correct Booth Rules

| Q₀Q₋₁ | Operation |
| --- | --- |
| 00 | No operation |
| 11 | No operation |
| 01 | A = A + M |
| 10 | A = A − M |

After each operation:

Arithmetic Right Shift

Common Errors

1. Incorrect Bit-Pair Checking

Using

Q1Q0

instead of

Q0Q−1

produces incorrect operations.

2. Logical Shift Instead of Arithmetic Shift

Incorrect:

01010010

Correct arithmetic shift preserves the sign bit.

Negative numbers require:

11101001

not

01101001

3. Incorrect Sign Extension

The sign bit must be copied during shifts.

Without sign extension:

Negative numbers become positive.

4. Wrong 2's Complement

Incorrect subtraction causes all later steps to fail.

Corrections

- Always compare Q₀Q₋₁.

- Use arithmetic right shift.

- Preserve the sign bit.

- Verify the 2's complement of negative operands.

- Repeat exactly n iterations for n-bit numbers.

4. Debugging Restoring Division

Problem

Restoring division performs unnecessary restore operations.

Workflow

For every iteration:

- Shift left

- Subtract divisor

- If remainder is negative

Restore remainder

- Set quotient bit

Inefficiency

Whenever subtraction produces a negative remainder:

Restore

means

Subtract

+

Add back

Two arithmetic operations are required.

This increases execution time.

Optimized Solution: Non-Restoring Division

Instead of restoring immediately:

- If remainder is positive

Subtract divisor

- If remainder is negative

Add divisor in the next iteration

Only one final correction is required if the remainder is negative.

Comparison

| Restoring | Non-Restoring |
| --- | --- |
| Frequent restoration | No immediate restoration |
| More additions | Fewer additions |
| Higher execution time | Lower execution time |
| Lower efficiency | Higher efficiency |

Performance Improvement

- Fewer arithmetic operations.

- Reduced execution cycles.

- Better hardware utilization.

- Preferred in modern processors.

5. Debugging IEEE 754 Floating-Point Addition

Problem

Adding a very small number to a very large number produces inaccurate results.

Example:

100000000 + 0.000001

Step 1: Exponent Alignment

Floating-point numbers must have the same exponent.

The smaller number's mantissa is shifted right.

Example:

Large exponent

1.010101 × 2²⁷

Small exponent

1.000000 × 2⁻²⁰

The small mantissa is shifted many positions.

Most bits disappear.

Step 2: Mantissa Addition

After shifting:

Small mantissa ≈ 000000000001

Adding it to the large mantissa changes almost nothing.

Step 3: Normalization

After addition:

- Normalize the result.

- Adjust the exponent if necessary.

Step 4: Rounding

IEEE 754 rounds the mantissa.

Lost bits are discarded.

This introduces small errors.

Causes of Inaccuracy

- Large exponent difference.

- Loss of significant digits during alignment.

- Rounding after normalization.

- Limited 23-bit mantissa in single precision.

Optimization Techniques

1. Use Double Precision

| Single Precision | Double Precision |
| --- | --- |
| 32 bits | 64 bits |
| 23-bit mantissa | 52-bit mantissa |
| Lower accuracy | Higher accuracy |

Double precision significantly reduces rounding and precision loss.

2. Rearrange Computations

Add numbers of similar magnitude first.

Example:

(a + b) + c

instead of

a + (b + c)

when a is much larger than b and c.

3. Compensated Summation

Use algorithms such as Kahan Summation to reduce accumulated rounding errors.

4. Scale Values

Normalize or scale inputs before performing arithmetic to reduce exponent differences.

5. Avoid Repeated Conversions

Minimize unnecessary conversions between floating-point formats, as repeated conversions can introduce additional rounding errors.