1. Smartphone Performance Optimization

Problem Analysis

The smartphone experiences lag while switching between applications. This usually occurs because the processor frequently accesses slower memory when application data is not available in the CPU cache.

The memory hierarchy can be represented as:

CPU

│

┌──▼──┐

│ L1  │  Fastest, smallest

└──┬──┘

│

┌──▼──┐

│ L2  │

└──┬──┘

│

┌──▼──┐

│ L3  │

└──┬──┘

│

┌──▼──┐

│DRAM │  Large, slower

└─────┘

Role of Each Memory

| Memory | Characteristics | Role |
| --- | --- | --- |
| L1 Cache | Very fast, very small | Stores frequently used instructions/data |
| L2 Cache | Fast, larger than L1 | Supplies data when L1 misses |
| L3 Cache | Larger, slower than L2 | Shared cache for multiple CPU cores |
| DRAM | Large capacity, higher latency | Stores active applications and OS data |

When switching applications, the required data may not be present in L1/L2/L3. The processor then accesses DRAM, causing increased latency.

Proposed Hierarchical Redesign

CPU Core

│

▼

Large/optimized L1

│

▼

Optimized private L2

│

▼

Shared L3

│

▼

High-bandwidth LPDDR

│

▼

Storage

Improvements

- Increase the effective L2/L3 cache capacity where power and die-area budgets permit.

- Use high-bandwidth, low-power LPDDR memory.

- Improve cache replacement policies.

- Use prefetching to predict application data requirements.

- Reduce unnecessary movement of data between DRAM and cache.

- Use memory compression where appropriate to reduce DRAM traffic.

Expected Result

More application data remains close to the CPU, reducing DRAM accesses. This decreases memory latency and improves application-switching responsiveness.

2. Cloud Server Memory Bottleneck

Problem Analysis

A cloud server experiences high latency during database queries. Two important sources of delay are:

- Cache misses

- Virtual-memory page faults

These mechanisms operate at different levels.

Cache Hierarchy

CPU

│

├── L1 Cache

│

├── L2 Cache

│

└── L3 Cache

│

▼

DRAM

Caches store frequently accessed data close to the processor.

A cache hit provides very low latency, while a cache miss requires accessing a lower level.

Virtual Memory Paging

Virtual memory allows processes to use a virtual address space larger than physical RAM.

CPU

│

▼

Virtual Address

│

▼

MMU / TLB

│

├── Page in DRAM

│

└── Page Fault

│

▼

Storage

A page fault is particularly expensive because data may need to be fetched from storage.

Cache vs Virtual Memory

| Feature | Cache Hierarchy | Virtual Memory |
| --- | --- | --- |
| Purpose | Reduce CPU memory latency | Extend/manage physical memory |
| Main hardware | SRAM-based caches | DRAM + storage |
| Normal latency | Very low | Higher |
| Unit of transfer | Cache line | Memory page |
| Miss penalty | Moderate | Potentially very high |
| Main benefit | Faster repeated access | Memory isolation and larger address space |

Recommended Improvements

1. Increase Effective Cache Capacity

Use processors with larger L2/L3 caches where the database workload benefits from them.

2. Increase DRAM Capacity

Insufficient RAM causes more paging. Keeping database working sets in DRAM avoids expensive storage accesses.

3. Use High-Performance DRAM

Higher-bandwidth DDR-class server memory can improve memory throughput for memory-intensive workloads.

4. Reduce Page Faults

Allocate sufficient RAM for the database buffer pool and frequently accessed working sets.

5. Improve TLB Performance

Large pages can reduce the number of TLB entries needed for large memory regions, potentially reducing address-translation overhead.

6. Use Fast NVMe Storage

Storage is still much slower than DRAM, but fast NVMe SSDs can reduce the penalty when paging or database I/O is unavoidable.

Recommended Hierarchy

CPU

↓

L1

↓

L2

↓

Large L3

↓

High-capacity DRAM

↓

NVMe SSD

Conclusion: Cache optimization should be the first priority for CPU-side latency, while sufficient DRAM capacity should be used to prevent database working sets from being paged to storage.

3. Autonomous Vehicle Data Processing

Problem Analysis

An autonomous vehicle continuously processes:

- Camera frames

- LiDAR data

- Radar data

- GPS information

- Vehicle-control data

- AI inference results

The memory system must provide low latency and high bandwidth while maintaining reliability.

SRAM

SRAM is:

- Very fast

- Low latency

- Expensive per bit

- Relatively low density

- Commonly used for CPU caches and small buffers

DRAM

DRAM provides:

- Much higher capacity than SRAM

- High bandwidth

- Lower cost per bit

- Higher latency than SRAM

It is suitable for large sensor buffers and AI workloads.

MRAM

MRAM is a non-volatile memory technology with:

- Non-volatility

- Good endurance

- Low standby power

- Fast access compared with many conventional non-volatile memories

It can be useful for persistent configuration, firmware-related data, and selected high-reliability applications.

Proposed Memory Hierarchy

CPU / AI Accelerator

│

▼

SRAM / Cache

│

▼

High-Speed DRAM

│

┌─────────┴─────────┐

▼                   ▼

Sensor Buffers        AI Data

│

▼

MRAM

│

▼

Persistent Storage

Recommended Allocation

| Memory | Suggested Usage |
| --- | --- |
| SRAM | CPU cache, control loops, frequently accessed sensor data |
| DRAM | Camera/LiDAR buffers, AI models, intermediate tensors |
| MRAM | Persistent configuration, critical parameters, selected firmware/data |

Optimization Techniques

Double Buffering

While one sensor buffer is being processed, another buffer receives new data.

Buffer A → Processing

Buffer B → Sensor acquisition

Then swap.

This improves continuous data flow.

DMA

Direct Memory Access can transfer sensor data directly into DRAM without requiring the CPU to handle every individual transfer.

Memory Partitioning

Reserve high-priority memory regions for safety-critical control data and allocate separate regions for high-volume sensor processing.

Result

The proposed hierarchy places latency-sensitive data in SRAM, high-volume sensor and AI data in DRAM, and persistent information in MRAM. This provides a balance between latency, bandwidth, capacity, power consumption, and reliability.

4. AI Inference Edge Device

Problem Analysis

An edge AI device needs:

- Fast model loading

- Low inference latency

- Low power consumption

- Sufficient memory capacity

The three technologies have different characteristics.

NAND Flash

NAND Flash is:

- Non-volatile

- High density

- Relatively inexpensive

- Slower than DRAM

- Suitable for storing complete AI models

DRAM

DRAM is:

- Volatile

- Much faster than NAND Flash

- High bandwidth

- Higher power consumption than non-volatile storage

- Suitable for active model execution

RRAM

Resistive RAM (RRAM) is a non-volatile memory technology that can provide high density and potentially low-power access. It is attractive for edge-AI architectures, including designs that seek to reduce data movement between memory and compute.

Proposed Hierarchy

AI Accelerator

│

▼

On-chip SRAM

│

▼

DRAM / RRAM

│

▼

NAND Flash

NAND Flash

Stores:

- AI models

- Operating system

- Applications

- Large datasets

DRAM

Stores:

- Currently executing model

- Activations

- Intermediate tensors

- Runtime data

RRAM

Can be used for:

- Frequently used model parameters

- Non-volatile AI storage

- Memory-centric/in-memory computing designs where supported

Model Loading Process

NAND Flash

│

│ Model loading

▼

DRAM / RRAM

│

▼

SRAM / AI Accelerator

│

▼

Inference

The complete model does not need to remain in SRAM because SRAM has limited capacity.

Recommended Design

For a conventional edge device:

Small SRAM

↓

Low-power DRAM

↓

High-capacity NAND Flash

For a specialized AI accelerator, RRAM can be incorporated closer to the compute units to reduce parameter movement.

Result

NAND Flash provides high-capacity persistent storage, DRAM provides high-bandwidth runtime memory, and RRAM can provide non-volatile storage and potentially reduce data movement in specialized AI architectures. The hierarchy therefore balances speed, capacity, and power consumption.

5. High-Performance Gaming Console

Problem Analysis

The gaming console experiences frame drops when loading large scenes.

A modern game may need to access:

- Textures

- 3D models

- Shader data

- Geometry

- Animation data

- World-state information

The main issue is often insufficient memory bandwidth, high latency, or excessive data movement rather than simply cache capacity.

L3 Cache vs DRAM

L3 Cache

Advantages:

- Very low latency compared with DRAM

- Keeps frequently reused data close to CPU cores

- Reduces DRAM traffic

Limitations:

- Much smaller capacity

- Expensive in silicon area

- Cannot store an entire large game scene

DRAM

Advantages:

- Much larger capacity

- High bandwidth

- Suitable for large game assets and working sets

Limitations:

- Higher latency than cache

- Consumes significant memory bandwidth

- Accesses can compete among CPU, GPU, and other devices

Comparison

| Feature | L3 Cache | DRAM |
| --- | --- | --- |
| Capacity | Small | Large |
| Latency | Very low | Higher |
| Bandwidth | High locally | Very high system bandwidth |
| Cost per bit | High | Lower |
| Main use | Frequently reused data | Large working datasets |
| Entire scene storage | No | Yes |

Proposed Hierarchy

CPU

│

▼

L1 Cache

│

▼

L2 Cache

│

▼

Large Shared L3 Cache

│

▼

High-Bandwidth DRAM

│

▼

High-Speed SSD

For a gaming console, the GPU should also have appropriate local/cache memory structures because rendering workloads are highly bandwidth-sensitive.

Enhancements

1. Increase Effective L3 Capacity

A larger or better-managed L3 cache can keep frequently reused CPU-side game data close to the processor.

2. Use High-Bandwidth DRAM

Higher memory bandwidth allows large texture, geometry, and scene data transfers to occur faster.

3. Improve Prefetching

Predictively load frequently required data before the CPU/GPU requests it.

4. Use DMA

Directly transfer game assets between storage and memory without unnecessarily involving the CPU.

5. Asset Streaming

Instead of loading an entire world simultaneously:

SSD

│

▼

DRAM

│

▼

GPU/CPU

Load only the assets needed for the current scene.

6. Compression

Texture and asset compression reduces the amount of data that must be transferred through the memory hierarchy.

Example

Suppose a scene requires 2 GB of data.

A small L3 cache cannot contain the entire scene, so:

L3 → Frequently reused CPU data

DRAM → Active scene data

SSD → Remaining game assets

This prevents the system from repeatedly fetching large amounts of data from storage.

Result

Increasing L3 cache can reduce CPU-side memory latency, but DRAM bandwidth is generally the more important factor for large scene and GPU workloads. The best solution is therefore a balanced hierarchy combining effective caches, high-bandwidth DRAM, fast storage, asset streaming, prefetching, and compression.

Overall Comparison

| Scenario | Primary Bottleneck | Best Memory Strategy |
| --- | --- | --- |
| Smartphone | App-switching latency | Optimized L1/L2/L3 + high-bandwidth LPDDR |
| Cloud Server | Database memory latency/paging | Large cache + sufficient DRAM + fast NVMe |
| Autonomous Vehicle | Real-time sensor bandwidth/latency | SRAM + high-bandwidth DRAM + MRAM |
| Edge AI | Model loading + power | SRAM + DRAM/RRAM + NAND Flash |
| Gaming Console | Scene streaming/bandwidth | Cache + high-bandwidth DRAM + fast SSD |

Final Conclusion

A memory hierarchy works because different memory technologies provide different trade-offs between latency, bandwidth, capacity, cost, volatility, and power consumption.

The general design principle is:

Fastest

▲

SRAM/Cache

│

DRAM

│

Non-volatile Memory

│

Flash/SSD

▼

Slowest

Small Capacity → Large Capacity

Frequently accessed and latency-sensitive data should be kept as close to the processor as possible, while large and persistent datasets should be placed in higher-capacity memory technologies.