1. I/O Communication Mechanism for a Real-Time Embedded System

Given Constraints

- CPU utilization < 15%

- Multiple sensors generate interrupts every 1 ms

- Vectored interrupt handling

- Choose between memory-mapped and I/O-mapped I/O for low latency

Proposed Architecture

┌───────────────┐

│    Sensors    │

│ S1 S2 S3 ...  │

└───────┬───────┘

│ Interrupts

▼

┌──────────────────┐

│ Interrupt Ctrl.  │

│ (Vectored IRQ)   │

└────────┬─────────┘

│ Vector

▼

┌──────────────────┐

│   CPU / RTOS     │

│ High Priority ISR│

└────────┬─────────┘

│

▼

┌──────────────────┐

│ Memory-Mapped    │

│ Sensor Registers │

└────────┬─────────┘

│

▼

Memory

Design

1. Vectored interrupts

Each sensor is assigned a unique interrupt vector.

For example:

| Device | Interrupt Vector | Priority |
| --- | --- | --- |
| Sensor 1 | 0x20 | High |
| Sensor 2 | 0x21 | High |
| Sensor 3 | 0x22 | High |
| Sensor 4 | 0x23 | Medium |

When an interrupt occurs, the CPU directly jumps to the corresponding ISR instead of searching for the source.

2. Memory-mapped I/O

Memory-mapped I/O is selected because sensor registers can be accessed using normal load/store instructions.

CPU → Load/Store → Sensor Register

This provides a simple and generally low-overhead interface and avoids requiring a separate I/O instruction set.

3. Short ISR

The ISR should only:

- Identify the sensor through the interrupt vector.

- Read the required register.

- Store the data in a buffer.

- Clear the interrupt.

- Return immediately.

Long processing should be performed by a lower-priority task.

CPU Utilization

If an ISR takes approximately 10 µs and an interrupt arrives every 1 ms:

Even with several sensors, the ISR must be optimized so that the combined utilization remains below 15%.

Result

Vectored + memory-mapped I/O + short ISRs provides low interrupt latency and keeps CPU utilization below the required limit.

2. High-Speed I/O Subsystem Using NVMe and PCIe

Given Constraints

- Storage throughput > 3 GB/s

- Must use NVMe over PCIe

- CPU should not perform bulk data transfer

- DMA with interrupt notification

Proposed Architecture

┌──────────┐

│   CPU    │

└────┬─────┘

│ Commands

▼

┌──────────────┐

│ NVMe Driver  │

└──────┬───────┘

│

PCIe │

▼

┌─────────────────┐

│   NVMe SSD      │

└────────┬────────┘

│

│ DMA

▼

┌─────────────────┐

│ Main Memory     │

│ DMA Buffers     │

└─────────────────┘

▲

│ Completion IRQ

│

┌─────┴──────┐

│ Interrupt  │

│ Controller │

└────────────┘

Operation

- CPU prepares an NVMe command.

- Command is placed in the NVMe submission queue.

- NVMe controller accesses the queue through DMA.

- SSD transfers data directly to system memory.

- CPU does not copy the bulk data.

- After completion, the NVMe controller generates an interrupt.

- CPU processes the completion entry.

DMA Transfer

Without DMA:

SSD → CPU → Memory

The CPU becomes a bottleneck.

With DMA:

SSD ───────────────→ Memory

DMA transfer

The CPU only handles commands and completion notifications.

Throughput

PCIe should provide sufficient bandwidth for the target of >3 GB/s. For example, a PCIe configuration with adequate lane width and generation speed should be selected so that protocol overhead still leaves more than 3 GB/s usable throughput.

Result

NVMe + PCIe + DMA + interrupt completion provides high throughput while minimizing CPU utilization.

3. Multi-Device Communication Architecture

Given Constraints

- 6 USB peripherals

- 2 Thunderbolt high-speed devices

- Avoid interrupt starvation

- Hardware and software interrupts

- Fair bus arbitration

Proposed Architecture

USB Device 1 ─┐

USB Device 2 ─┤

USB Device 3 ─┤

USB Device 4 ─┤

USB Device 5 ─┤

USB Device 6 ─┘

│

USB Controller

│

▼

┌───────────────┐

│ I/O Controller│

└───────┬───────┘

│

▼

PCIe

│

┌────────┴────────┐

│                 │

▼                 ▼

Thunderbolt 1       Thunderbolt 2

│                 │

└────────┬────────┘

▼

┌───────────────┐

│ Interrupt     │

│ Controller    │

└───────┬───────┘

▼

CPU

Interrupt Management

Hardware interrupts are generated by USB and Thunderbolt controllers when an important event occurs.

Software interrupts can be used for deferred processing:

Hardware IRQ

↓

Short ISR

↓

Set software interrupt/task

↓

Process data

This prevents lengthy processing inside the hardware ISR.

Preventing Interrupt Starvation

Use:

- Interrupt priorities

- Interrupt masking only for very short periods

- Deferred interrupt processing

- Interrupt coalescing where supported

- Separate queues for high- and low-priority devices

Example:

| Device | Priority |
| --- | --- |
| Thunderbolt 1 | Very High |
| Thunderbolt 2 | Very High |
| USB 1–2 | High |
| USB 3–6 | Medium |

Fair Bus Arbitration

A round-robin arbitration mechanism can be used.

USB1 → USB2 → USB3 → USB4

↑                    ↓

USB6 ← USB5 ←────────

Each device receives a fair opportunity to use the bus.

For high-speed devices, bandwidth reservations or weighted arbitration can be added without allowing USB devices to starve.

Result

A combination of priority-based interrupts + software interrupts + round-robin/weighted bus arbitration prevents starvation while maintaining high performance for Thunderbolt devices.

4. Data Acquisition System at 500 MB/s

Given Constraints

- Continuous stream: 500 MB/s

- Minimal CPU intervention

- Compare DMA and interrupt-driven I/O

- Memory-mapped device registers

Proposed Architecture

┌──────────────────┐

│ Data Acquisition │

│     Device       │

└────────┬─────────┘

│

Memory-Mapped Registers

│

▼

┌───────────────┐

│ DMA Controller│

└───────┬───────┘

│

High-speed DMA

│

▼

┌───────────────┐

│ Ring Buffer   │

│    Memory     │

└───────┬───────┘

│

▼

CPU

Process completed

buffers

Memory-Mapped I/O

The acquisition device's control registers are mapped into the CPU address space.

For example:

0x80000000 → Control Register

0x80000004 → Status Register

0x80000008 → DMA Address

0x8000000C → DMA Length

The CPU configures these registers using normal memory access instructions.

DMA vs Interrupt-Driven I/O

| Feature | Interrupt-Driven I/O | DMA |
| --- | --- | --- |
| CPU involvement | High | Low |
| Transfer mechanism | CPU copies data | DMA copies data |
| Interrupt frequency | Very high | Low |
| CPU overhead | High | Low |
| Suitable for 500 MB/s | No | Yes |
| Continuous streaming | Difficult | Excellent |
| Throughput | Lower | High |

DMA Operation

The CPU initially configures:

Source = Acquisition Device

Destination = RAM buffer

Length = Buffer size

Then DMA transfers:

Device ─────────────→ RAM

500 MB/s

When a buffer is full:

DMA → Interrupt → CPU

The CPU processes the completed buffer while DMA fills another buffer.

Double Buffering

DMA

│

▼

┌────────────┐

│ Buffer A   │ ← CPU processes

└────────────┘

┌────────────┐

│ Buffer B   │ ← DMA fills

└────────────┘

After Buffer B is filled, the roles switch.

Result

For a continuous 500 MB/s stream, DMA with double/ring buffering is clearly preferable because CPU intervention occurs only at buffer boundaries rather than for every data transfer.

5. Interrupt Handling Mechanism

Given Constraints

- Nested interrupts

- Maximum latency < 10 µs

- Vectored interrupts for high-priority devices

- Software interrupts for background tasks

Proposed Architecture

Devices

│

▼

┌─────────────────┐

│ Interrupt       │

│ Controller      │

└────────┬────────┘

│

┌────────┴────────┐

│                 │

High Priority       Low Priority

Hardware IRQ        Software IRQ

│                 │

▼                 ▼

Vectored ISR       Background Task

│

▼

CPU / RTOS

Interrupt Priority

Example:

| Interrupt | Type | Priority |
| --- | --- | --- |
| Emergency sensor | Hardware/vector | 1 |
| Communication device | Hardware/vector | 2 |
| Timer | Hardware | 3 |
| Data processing | Software | 4 |
| Logging | Software | 5 |

Lower numerical value represents higher priority.

Nested Interrupt Operation

Suppose the CPU is executing a low-priority ISR:

Low-priority ISR

│

│ High-priority IRQ arrives

▼

Save current context

│

▼

High-priority ISR

│

▼

Restore context

│

▼

Continue low-priority ISR

This ensures critical devices are serviced immediately.

Vectored Interrupts

Each high-priority interrupt has a dedicated vector:

Vector 0x20 → Timer ISR

Vector 0x21 → Sensor ISR

Vector 0x22 → Communication ISR

The processor directly jumps to the required handler.

Software Interrupts

Background operations such as:

- Logging

- Data processing

- Statistics

- Non-critical communication

can be deferred to software interrupts/tasks.

Hardware IRQ

↓

Short ISR

↓

Set software interrupt

↓

Background processing

Achieving <10 µs Latency

To meet the maximum latency requirement:

- Use vectored interrupts.

- Keep ISRs extremely short.

- Use priority-based interrupt nesting.

- Avoid long critical sections.

- Disable interrupts only for essential operations.

- Use hardware-assisted interrupt controllers.

- Defer non-critical processing to software interrupts/tasks.

Result

The architecture provides nested, priority-based vectored interrupts for critical devices while moving background work to software interrupts, allowing the system to target an interrupt latency of less than 10 µs.