1. DMA-Based Data Transfer Algorithm

ALGORITHM DMA_Data_Transfer

INPUT:

IO_Device

Memory_Buffer

Data_Size

BEGIN

Initialize DMA_Controller

Configure DMA_Controller:

Source      ← IO_Device

Destination ← Memory_Buffer

Transfer_Size ← Data_Size

Direction ← IO_TO_MEMORY

Interrupt_Enable ← TRUE

Clear DMA_Status

Enable DMA_Interrupt

Start DMA_Transfer

WHILE DMA_Transfer_Completed = FALSE DO

CPU performs other tasks

END WHILE

ON DMA_Interrupt:

Save CPU_Context

Read DMA_Status

IF DMA_Status = TRANSFER_COMPLETE THEN

Clear DMA_Interrupt

Set Transfer_Status ← SUCCESS

ELSE IF DMA_Status = TRANSFER_ERROR THEN

Clear DMA_Interrupt

Set Transfer_Status ← ERROR

Handle_DMA_Error()

END IF

Restore CPU_Context

Return_From_Interrupt

RETURN Transfer_Status

END

Working

I/O Device

│

│ DMA

▼

Memory Buffer

│

▼

DMA Completion Interrupt

│

▼

CPU handles only completion/error

Advantage: The CPU does not copy each byte/word; DMA performs the bulk transfer.

2. Vectored Interrupt Handling System

ALGORITHM Interrupt_Handler

INPUT:

Interrupt_Controller

Vector_Table

BEGIN

Initialize Vector_Table

Set Interrupt_Priorities

Enable Global_Interrupts

WHILE System_Running = TRUE DO

IF Interrupt_Request_Exists() THEN

Vector_Number ← Get_Interrupt_Vector()

Priority ← Get_Priority(Vector_Number)

IF Priority >= Current_Priority THEN

Save_CPU_Context()

Current_Priority ← Priority

ISR_Address ← Vector_Table[Vector_Number]

Jump_To(ISR_Address)

Execute ISR:

Identify_Device(Vector_Number)

IF Device = Timer THEN

Timer_ISR()

ELSE IF Device = Sensor THEN

Sensor_ISR()

ELSE IF Device = Network THEN

Network_ISR()

ELSE

Default_ISR()

END IF

Clear_Interrupt(Vector_Number)

Restore_CPU_Context()

Current_Priority ← Previous_Priority

Return_From_Interrupt()

ELSE

Queue_Interrupt(Vector_Number)

END IF

END IF

END WHILE

END

Interrupt Flow

Device

↓

Interrupt Controller

↓

Vector Number

↓

Vector Table

↓

Correct ISR

↓

Service Device

↓

Clear Interrupt

↓

Return to Program

This avoids searching through all devices because the vector directly identifies the required service routine.

3. Dynamic Selection of Memory-Mapped I/O and I/O-Mapped I/O

ALGORITHM Select_IO_Mechanism

INPUT:

Latency_Requirement

Bandwidth_Requirement

Device_Type

OUTPUT:

Selected_IO_Method

BEGIN

MMIO_Score ← 0

IOMM_Score ← 0

IF Latency_Requirement = HIGH THEN

MMIO_Score ← MMIO_Score + 3

ELSE

IOMM_Score ← IOMM_Score + 1

END IF

IF Bandwidth_Requirement = HIGH THEN

MMIO_Score ← MMIO_Score + 3

ELSE

IOMM_Score ← IOMM_Score + 1

END IF

IF Device_Type = MEMORY_LIKE_DEVICE THEN

MMIO_Score ← MMIO_Score + 2

END IF

IF Device_Type = LEGACY_IO_DEVICE THEN

IOMM_Score ← IOMM_Score + 2

END IF

IF MMIO_Score > IOMM_Score THEN

Selected_IO_Method ← MEMORY_MAPPED_IO

ELSE IF IOMM_Score > MMIO_Score THEN

Selected_IO_Method ← IO_MAPPED_IO

ELSE

Selected_IO_Method ← MEMORY_MAPPED_IO

END IF

RETURN Selected_IO_Method

END

Selection Logic

Requirements

│

┌─────────┴─────────┐

│                   │

High latency         High bandwidth

sensitivity           requirement

│                   │

└─────────┬─────────┘

▼

Memory-Mapped I/O

For conventional high-performance embedded/device interfaces, MMIO is generally preferred because device registers can be accessed through normal load/store operations.

4. Bus Arbitration Algorithm for PCIe, USB and Thunderbolt

ALGORITHM Bus_Arbitration

INPUT:

Device_Request_List

BEGIN

Initialize Device_Queues

Set Priority:

Thunderbolt ← HIGH

PCIe        ← HIGH

USB         ← MEDIUM

WHILE System_Running = TRUE DO

Collect_All_Requests()

Remove_Inactive_Devices()

FOR each Device IN Device_Request_List DO

Calculate_Request_Priority(Device)

END FOR

Selected_Device ← NULL

// First serve high-priority devices

FOR each Device IN Device_Request_List DO

IF Device.Request = TRUE AND

Device.Priority = HIGH THEN

IF Selected_Device = NULL OR

Device.Wait_Time > Selected_Device.Wait_Time THEN

Selected_Device ← Device

END IF

END IF

END FOR

// If no high-priority request exists,

// use round-robin for fairness

IF Selected_Device = NULL THEN

Selected_Device ←

Round_Robin_Select(Device_Request_List)

END IF

IF Selected_Device ≠ NULL THEN

Grant_Bus(Selected_Device)

Transfer_Data(Selected_Device)

Update_Bandwidth(Selected_Device)

Increase_Wait_Time_For_Others()

Release_Bus(Selected_Device)

Move_Round_Robin_Pointer()

END IF

END WHILE

END

Arbitration Concept

Bus Requests

│

┌─────────┼─────────┐

▼         ▼         ▼

PCIe       USB    Thunderbolt

│         │         │

└─────────┼─────────┘

▼

Priority Check

│

▼

Fair Arbitration

│

▼

Bus Grant / Transfer

Fairness

To prevent starvation, the algorithm increases the waiting time of devices that have not received bus access.

A device that waits too long can have its effective priority increased.

5. Hybrid I/O Communication Mechanism

Concept

- Low-speed devices → Polling

- High-speed devices → DMA + Interrupts

ALGORITHM Hybrid_IO_Manager

INPUT:

Device_List

BEGIN

FOR each Device IN Device_List DO

Speed ← Determine_Device_Speed(Device)

IF Speed = LOW THEN

Device.Mode ← POLLING

ELSE IF Speed = HIGH THEN

Device.Mode ← DMA_INTERRUPT

Configure_DMA(Device)

Enable_Device_Interrupt(Device)

END IF

END FOR

WHILE System_Running = TRUE DO

FOR each Device IN Device_List DO

IF Device.Mode = POLLING THEN

IF Device_Status(Device) = DATA_AVAILABLE THEN

Data ← Read_Data(Device)

Store_Data(Data)

END IF

END IF

END FOR

// High-speed devices operate using DMA

// without continuous CPU intervention

IF DMA_Interrupt_Received() THEN

Device ← Identify_DMA_Device()

Status ← Read_DMA_Status(Device)

IF Status = COMPLETE THEN

Mark_Buffer_Ready(Device)

Clear_DMA_Interrupt(Device)

ELSE IF Status = ERROR THEN

Handle_DMA_Error(Device)

END IF

END IF

END WHILE

END

Hybrid Architecture

I/O Devices

│

┌──────────┴──────────┐

│                     │

Low-Speed              High-Speed

Devices                Devices

│                     │

▼                     ▼

POLLING              DMA Controller

│

▼

Memory

│

▼

Interrupt

│

▼

CPU

Example

| Device | Speed | Method |
| --- | --- | --- |
| Keyboard | Low | Polling |
| Temperature Sensor | Low | Polling |
| Mouse | Low/Medium | Polling |
| Network Adapter | High | DMA + Interrupt |
| NVMe SSD | Very High | DMA + Interrupt |
| Data Acquisition Device | High | DMA + Interrupt |

Main Advantage

The hybrid mechanism avoids wasting CPU cycles polling high-speed devices while also avoiding unnecessary interrupt overhead for very slow devices.

Overall strategy: